Wi-Fi Jamming – Technical Report

1. Introduction

Wi-Fi has become an essential communication medium in modern digital systems. It enables wireless connectivity between devices such as smartphones, laptops, IoT devices, and access points. However, because Wi-Fi operates on open, unlicensed radio frequencies, it is vulnerable to interference—both accidental and intentional.
This report discusses Wi-Fi jamming, the concept, how it works, and the ethical/legal implications. It also analyzes a Python-based Wi-Fi scanner/jammer script purely for educational and defensive awareness.

2. What is Wi-Fi Jamming?

Wi-Fi jamming refers to the intentional disruption of wireless communication by sending excessive or specially crafted packets that interfere with normal network operation.

2.1 How Wi-Fi Works (Simplified)

- Devices communicate using IEEE 802.11 protocol

- Access Points (APs) broadcast beacons and manage communication

- Clients send/receive data using shared radio channels

- Frames such as beacon frames, management frames, and control frames manage connectivity

2.2 What Is Jamming?

In Wi-Fi, jamming occurs when:

- An attacker transmits large amounts of noise

- Or sends management frames (like deauthentication frames) to disconnect users

These signals confuse client devices, preventing normal communication.

3. Types of Wi-Fi Jamming (Overview)

1. Noise Jamming

Floods the channel with random signals → AP and clients cannot communicate.

2. Deauthentication Attack

Exploits the fact that Wi-Fi management frames are not always encrypted.
Clients receive fake “deauth” packets and disconnect from the network.

3. Beacon Flooding

Fake access points are created to overwhelm the wireless environment.

4. Channel Saturation

Rapid channel switching and sending packets to multiple frequencies.

Note:
These methods are illegal outside authorized security testing environments.

4. Overview of the Provided Script (Educational Explanation)

Your Python script uses Scapy, threading, and monitor mode interfaces to:

- Scan nearby Wi-Fi networks

- Identify APs and clients

- Allow user selection

- Send deauthentication frames to targeted clients

Below is a high-level explanation (without harmful detail):

4.1 Channel Hopping

def channelHopper():

...

- Randomly switches Wi-Fi channels (1–11)

- Helps capture networks operating on different frequencies

- Runs in a separate thread so scanning continues smoothly

4.2 Packet Handler (Scanning Phase)

def handler(packet):

...

This function:

- Detects Beacon frames from APs

- Reads SSID, MAC address, and channel

- Filters noise depending on user choice

- Stores unique networks in a list

This is used for network discovery, not for attacking.

4.3 Finding Clients

def clientFilter(packet):

...

- Monitors packets flowing between AP and clients

- Identifies connected client MAC addresses

- Useful for analyzing network topology

4.4 Deauthentication Packet Sender

packet = RadioTap()/Dot11(...)/Dot11Deauth()

sendp(packet)

- Sends deauthentication frames

- This causes targeted devices to disconnect

- It is the core of Wi-Fi jamming

Important Note:
This part of the script is illegal to use on any network that you do not own or have explicit permission to test.
This is presented only for awareness in cybersecurity learning.

5. Ethical & Legal Considerations

Wi-Fi jamming is treated as a criminal offense in most countries including India, the US, UK, and the EU.

5.1 Illegal Under

- Telegraph Act / Wireless Act (India)

- FCC Regulations (USA)

- Computer Misuse Acts

- Cybercrime & IT laws worldwide

It can lead to:

- Heavy fines

- Seizure of equipment

- Criminal prosecution

- Imprisonment

5.2 When Is It Legal?

Only when:

- You own the network AND

- You are performing security auditing with permission

- In controlled lab or simulation environments

6. Defensive Applications (Why Learn About This?)

Learning Wi-Fi jamming is essential for cybersecurity research, because it helps in:

6.1 Testing Network Security

- Ensuring APs support 802.11w (Protected Management Frames)

- Validating WPA3 protections

- Ensuring hardware is resistant to deauth flooding

6.2 Developing Anti-Jamming Techniques

- Adaptive frequency hopping

- Jamming detection systems

- Network monitoring tools

6.3 Strengthening Network Infrastructure

- Detecting rogue access points

- Improving channel allocation

- Hardening corporate Wi-Fi

🛠️ Terminal Command Execution (Attack Demonstration)

These screenshots illustrate the technical steps involved in the network monitoring and Deauthentication attack.

| Image | Description and Contextual Meaning |
| --- | --- |
| USB Device Selection in VirtualBox | A dropdown menu showing the user selecting a USB device, specifically a Wi-Fi adapter (e.g., MediaTek 802.11 n WLAN). This is critical because specialized USB Wi-Fi adapters are required to support monitor mode and packet injection—features necessary for security research that are often unsupported by built-in Wi-Fi cards11111111. |
| airmon-ng start Command | A terminal screenshot showing the command sudo airmon-ng start wlan0. This command puts the wireless interface (wlan0) into monitor mode, preparing it to capture raw wireless packets, which is an essential first step for network analysis12. |
| airodump-ng Command | A terminal screenshot showing the command sudo airodump-ng wlan0mon. This tool scans the air for nearby Access Points (APs) and connected clients, showing information like BSSID, ESSID, and Channel13. |
| airodump-ng Output | The output of the airodump-ng command, which displays a list of detected Wi-Fi networks and associated client devices. The data includes the BSSID (MAC address of the AP), signal strength, channel (CH), and ESSID (network name). |
| aireplay-ng Deauthentication Command | A screenshot showing a terminal continuously sending Deauthentication packets using a command structure like sudo aireplay-ng -0 <count> -a <AP MAC> -c <Client MAC> wlan0mon. This is the direct execution of the Deauthentication attack, which forcibly disconnects legitimate users from networks14. The command window shows "Sending DeAuth (code 7) to broadcast," confirming the attack is running. |

🐛 Troubleshooting

- Error Report and Solution | This is a text slide listing common technical issues encountered during the project (e.g., adapter issues, monitor mode failing, library problems) and their corresponding solutions. It emphasizes that security research often involves troubleshooting and highlights the need to check adapter compatibility, ensure proper driver updates, and use isolated lab environments.

7. Conclusions

Wi-Fi jamming is a powerful but dangerous technique that disrupts wireless communication by exploiting vulnerabilities in the Wi-Fi protocol. While the technology is useful in cybersecurity research and academic environments, its misuse is illegal and unethical.

The provided script demonstrates how:

- Wi-Fi scanning

- Channel hopping

- Client detection

- Deauthentication frame broadcasting

work at a technical level, giving valuable insight into wireless security concepts.
However, this knowledge must always be applied responsibly, within legal boundaries, and for defensive cybersecurity purposes only.

ACKNOWLEGMENT:

Team Leader

- Shivang Mahawar

Members

- Ronak Sharma

- Bhavishya

- Shaurya Sharma

- Priyanshu Kumar